Content

2	Classes	2

2.1	ConnectorSpec	2

2.2	Connector_Pac	2

2.3	ControlConstructSpec	3

2.4	ControlConstruct_Pac	3

2.5	EquipmentTypeSpec	4

2.6	EquipmentType_Pac	5

2.7	HolderSpec	6

2.8	Holder_Pac	6

2.9	Protocol	7

2.10	ProtocolCollection	8

3	Data Types	9

3.1	InstanceIdentifier	9

4	Enumeration Types	9

4.1	ConnectorKindType	9

4.2	EquipmentCategory	10

4.3	ProtocolNameType	11

4.4	RestartType	11

5	Primitive Types	12

# Classes

## ConnectorSpec

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for ConnectorSpec

Table 1: Attributes for ConnectorSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _connector_Pac | Connector_Pac ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## Connector_Pac

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for Connector_Pac

Table 1: Attributes for Connector_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| connectorKind | ConnectorKindType NOT_YET_DEFINED | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Kind of physical connector as available at the outside of the device. |
| outsideLabel | String Label of the position not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | String, which is identifying the physical position of a connector (e.g. RJ45 connector, SFP, IF port) at the outside of the device. If the attribute is generally supported by the device, but there is actually no label at this connector, an empty string shall be represented. |
| sequenceId | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | All instances of Connector in the Equipment::connector attribute shall be identifiable by a locally unique 'sequenceId'. The 'sequenceId' should be unique for the list of Connector objects of the Equipment object. Gaps in the sequence would be legal. |

## ControlConstructSpec

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for ControlConstructSpec

Table 1: Attributes for ControlConstructSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _controlConstruct_Pac | ControlConstruct_Pac ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _protocolCollection | ProtocolCollection ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## ControlConstruct_Pac

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for ControlConstruct_Pac

Table 1: Attributes for ControlConstruct_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| externalLabel | String External label not yet defined. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | The externalLabel attribute is for the operator to persistently store an identifier from his documentation inside the device. |
| lastConfigChangeTimestamp | DateTime 2010-11-20T14:00:00+01:00 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | This attribute reports the timestamp of the last configuration-change. Configuration changes are AttributeValueChanges, ObjectDeletions and ObjectCreations. |
| deviceModelName | String Device model not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NA • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Name of the device model according to its official documentation. |

## EquipmentTypeSpec

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for EquipmentTypeSpec

Table 1: Attributes for EquipmentTypeSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _equipmentType_Pac | EquipmentType_Pac ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## EquipmentType_Pac

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for EquipmentType_Pac

Table 1: Attributes for EquipmentType_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| outsideLabel | String Label at the equipment not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Label, which is printed at the outside of the equipment. If the attribute is generally supported by the device, but there is actually no label at this equipment, an empty string shall be represented. |
| lctLabel | String Label on the LCT not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Identifies the type of equipment on the vendor's own management interface (e.g. on the Local Craft Terminal). |

## HolderSpec

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for HolderSpec

Table 1: Attributes for HolderSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _holder_Pac | Holder_Pac ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## Holder_Pac

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for Holder_Pac

Table 1: Attributes for Holder_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| vendorLabel | String Vendor label not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Identifier of the holder according to the currently valid, official documentation of the equipment. |
| outsideLabel | String Label of the position not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Label, which is printed at the outside of the device for identifying the physical position e.g. of a board inside the device. If the attribute is generally supported by the device, but there is actually no label at this holder, an empty string shall be represented. |
| sequenceId | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | All instances of Holder in the Equipment::containedHolder attribute shall be identifiable by a unique 'sequenceId'. The 'sequenceId' should be unique for the list of Holder objects of the Equipment object. Gaps in the sequence would be legal. |

## Protocol

Attachment point for technology specific attributes of protocols as LLDP and similar.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for Protocol

Table 1: Attributes for Protocol

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| protocolName | ProtocolNameType NOT_YET_DEFINED | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY |  |

## ProtocolCollection

List of protocols such as LLDP and similar.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for ProtocolCollection

Table 1: Attributes for ProtocolCollection

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _protocol | Protocol ./. | 0..* | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

# Data Types

## InstanceIdentifier

To be translated into the YANG built in datatype instance-identifier.

Applied Stereotypes:

# Enumeration Types

## ConnectorKindType

If required, this identity shall be referenced as a base identity for identities representing individual connector types as they are supported by the individual device. The additional identities referencing the CONNECTOR_KIND_TYPE shall be provided by vendors in a separate YANG module.

Contains Enumeration Literals:

- SC:

  - Subscriber Connector

- FIBRE_CHANNEL_STYLE_1:

  - Copper connector

- FIBRE_CHANNEL_STYLE_2:

  - Copper connector

- BNC:

  - Bayonet/Threaded Neill-Concelman

- FC:

  - Fibre Channel coax headers

- FIBER_JACK:

- LC:

  - Lucent Connector

- MT_RJ:

  - Mechanical Transfer - Registered Jack

- MU:

  - Multiple Optical

- SG:

- OPTICAL_PIGTAIL:

- MPO1x12:

  - Multifiber Parallel Optic

- MPO2x16:

  - Multifiber Parallel Optic

- HSSDC_II:

  - High Speed Serial Data Connector

- COPPER_PIGTAIL:

- RJ45:

  - 8P8C, according to Clause 3 and Figures 1 through 5 of IEC 60603-7

- NO_SEPERABLE_CONNECTOR:

- MXC2x16:

- ST:

  - according to IEC 60874-10:1992, also often called BFOC/2.5

- SMA:

- N:

- DIN_1.0_2.3:

- SCSI_50_PIN_1.27MM_PITCH:

- SCSI_FEMALE_68_PIN:

- D_SUB_FEMALE_15_PIN:

- RS-422:

- RS-232:

- RS-485:

- MDR68:

- TNC:

- FLANGE:

- NOT_YET_DEFINED:

## EquipmentCategory

Contains Enumeration Literals:

- BACKPLANE:

  - This identity is applicable if the hardware class is some sort of device for aggregating and forwarding networking traffic, such as a shared backplane in a modular ethernet switch.  Note that an implementation may model a backplane as a single physical component, which is actually implemented as multiple discrete physical components (within a chassis or stack).

- POWER_UNIT:

  - This identity is applicable if the hardware class is a power-supplying component.

- BATTERY:

  - This identity is applicable if the hardware class is some sort of battery.

- FAN:

  - This identity is applicable if the hardware class is a fan or other heat-reduction component.

- CENTRAL_PROCESSING_UNIT:

  - This identity is applicable if the hardware class is some sort of central processing unit.

- STORAGE_DRIVE:

  - This identity is applicable if the hardware class is some sort of component with data storage capability as its main functionality, e.g., hard disk drive (HDD), solid-state device (SSD), solid-state hybrid drive (SSHD), object storage device (OSD), or other.

- SENSOR:

  - This identity is applicable if the hardware class is some sort of sensor, such as a temperature sensor within a router chassis.

- MODEM:

  - Modulator-demodulator for converting digital data into analog signals for transmission and vice versa.

- PORT_EXPANSION_BOARD:

  - A port expansion board is an add-on hardware component that increases the number of available physical connectors and logical interfaces.

- OUTDOOR_UNIT:

  - Externally mounted hardware component that contains a radio frequency (RF) transceiver. The outdoor unit communicates with the indoor unit (IDU) via an IF interface.

- FULL_OUTDOOR_UNIT:

  - Device that contains at least one pair of modem and radio frequency (RF) transceiver and might also contain switching functionality. The full outdoor unit provides payload interfaces (e.g. according to IEEE 802.3) for communicating with diverse kinds of devices.

- NOT_YET_DEFINED:

## ProtocolNameType

List of Protocol names.

Contains Enumeration Literals:

- NOT_YET_DEFINED:

## RestartType

Contains Enumeration Literals:

- COLD:

  - Service affecting restart of the device or component.

- WARM:

  - Non-service affecting restart of the device or component.